EXPERIMENT 01

COGNITIVE RADIO

clc;

clear;

close all;

fs = 1000;

fPrimary = 100;

fSecondary = 200;

N = 1000;

t = (0:N-1)/fs;

threshold = 0.5;

%% =========================================================

% CASE 1 : PRIMARY USER ABSENT

% ==========================================================

primarySignal1 = zeros(1,N);

noise1 = 0.2*randn(1,N);

receivedSignal1 = primarySignal1 + noise1;

energy1 = mean(receivedSignal1.^2);

if energy1 < threshold

decision1 = 'CHANNEL FREE';

secondarySignal1 = sin(2*pi*fSecondary*t);

else

decision1 = 'CHANNEL OCCUPIED';

secondarySignal1 = zeros(1,N);

end

%% =========================================================

% CASE 2 : PRIMARY USER PRESENT

% ==========================================================

primarySignal2 = sin(2*pi*fPrimary*t);

noise2 = 0.2*randn(1,N);

receivedSignal2 = primarySignal2 + noise2;

energy2 = mean(receivedSignal2.^2);

if energy2 > threshold

decision2 = 'CHANNEL OCCUPIED';

secondarySignal2 = zeros(1,N);

else

decision2 = 'CHANNEL FREE';

secondarySignal2 = sin(2*pi*fSecondary*t);

end

%% =========================================================

% DISPLAY RESULTS

% ==========================================================

fprintf('\n========== CASE 1 : PRIMARY USER ABSENT ==========\n');

fprintf('Measured Energy : %.4f\n',energy1);

fprintf('Threshold       : %.2f\n',threshold);

fprintf('Decision        : %s\n',decision1);

if energy1 < threshold

disp('Secondary User : Spectrum Access Granted')

else

disp('Secondary User : Spectrum Occupied')

end

fprintf('\n========== CASE 2 : PRIMARY USER PRESENT =========\n');

fprintf('Measured Energy : %.4f\n',energy2);

fprintf('Threshold       : %.2f\n',threshold);

fprintf('Decision        : %s\n',decision2);

if energy2 > threshold

disp('Secondary User : Spectrum Occupied')

else

disp('Secondary User : Spectrum Access Granted')

end

%% =========================================================

% FIGURE 1 : CASE 1

% ==========================================================

figure('Name','CASE 1 : Energy < Threshold','NumberTitle','off');

subplot(5,1,1)

plot(t,primarySignal1,'b','LineWidth',1.5)

grid on

title('Primary User Signal (Absent)')

ylabel('Amplitude')

subplot(5,1,2)

plot(t,noise1,'m')

grid on

title('AWGN Channel')

ylabel('Amplitude')

subplot(5,1,3)

plot(t,receivedSignal1,'r')

grid on

title('Received Signal')

ylabel('Amplitude')

subplot(5,1,4)

bar([energy1 threshold])

set(gca,'XTickLabel',{'Measured Energy','Threshold'})

grid on

title('Energy Detection')

ylabel('Energy')

subplot(5,1,5)

plot(t,secondarySignal1,'g','LineWidth',1.5)

grid on

title('Secondary User Transmission')

xlabel('Time (s)')

ylabel('Amplitude')

%% =========================================================

% FIGURE 2 : CASE 2

% ==========================================================

figure('Name','CASE 2 : Energy > Threshold','NumberTitle','off');

subplot(5,1,1)

plot(t,primarySignal2,'b','LineWidth',1.5)

grid on

title('Primary User Signal (Present)')

ylabel('Amplitude')

subplot(5,1,2)

plot(t,noise2,'m')

grid on

title('AWGN Channel')

ylabel('Amplitude')

subplot(5,1,3)

plot(t,receivedSignal2,'r')

grid on

title('Received Signal')

ylabel('Amplitude')

subplot(5,1,4)

bar([energy2 threshold])

set(gca,'XTickLabel',{'Measured Energy','Threshold'})

grid on

title('Energy Detection')

ylabel('Energy')

subplot(5,1,5)

plot(t,secondarySignal2,'k','LineWidth',1.5)

grid on

title('Secondary User Blocked')

xlabel('Time (s)')

ylabel('Amplitude')

%% =========================================================

% FIGURE 3 : SNR, NOISE AND BER ANALYSIS

% ==========================================================

% SNR values to demonstrate

selectedSNR = [0 5 10 20];

% Signal used for SNR visualization

tBER = 0:1/1000:0.1;

cleanSignal = sin(2*pi*50*tBER);

signalPower = mean(cleanSignal.^2);

figure('Name','SNR Noise and BER Analysis','NumberTitle','off');

%% Plot received waveform for each SNR

for i = 1:length(selectedSNR)

snrLinear = 10^(selectedSNR(i)/10);

noisePower = signalPower/snrLinear;

noise = sqrt(noisePower)*randn(size(cleanSignal));

received = cleanSignal + noise;

subplot(5,1,i)

plot(tBER,received,'LineWidth',0.8)

hold on

plot(tBER,cleanSignal,'--','LineWidth',0.8)

grid on

ylabel('Amplitude')

if selectedSNR(i) == 0

title('SNR = 0 dB : Very High Noise Effect')

elseif selectedSNR(i) == 5

title('SNR = 5 dB : Moderate Noise Effect')

elseif selectedSNR(i) == 10

title('SNR = 10 dB : Low Noise Effect')

else

title('SNR = 20 dB : Very Low Noise Effect')

end

legend('Received','Clean','Location','eastoutside')

hold off

end

%% =========================================================

% BER CURVE

% ==========================================================

SNRdB = 0:0.1:20;

SNRlinear = 10.^(SNRdB/10);

% BER over AWGN

BER = 0.5*erfc(sqrt(SNRlinear));

% Important BER points

importantSNR = [0 5 10 20];

importantBER = 0.5*erfc(sqrt(10.^(importantSNR/10)));

% Minimum value for logarithmic display

importantBER(importantBER < 1e-12) = 1e-12;

subplot(5,1,5)

semilogy(SNRdB,BER,'LineWidth',1)

hold on

semilogy(importantSNR,importantBER, 'o','MarkerSize',5,'LineWidth',1)

grid on

grid minor

xlabel('SNR (dB)')

ylabel('BER')

title('SNR vs BER Performance')

xlim([0 20])

ylim([1e-12 1])

xticks([0 5 10 15 20])

hold off

%% =========================================================

% FINAL RESULT

% ==========================================================

fprintf('\n================ FINAL RESULT ================\n');

fprintf('CASE 1 : %s\n',decision1);

fprintf('CASE 2 : %s\n',decision2);

fprintf('\nSNR Analysis:\n');

fprintf('0 dB  : Very High Noise Effect\n');

fprintf('5 dB  : Moderate Noise Effect\n');

fprintf('10 dB : Low Noise Effect\n');

fprintf('20 dB : Very Low Noise Effect\n');

fprintf('\nAs SNR increases, noise effect and BER decrease.\n');

fprintf('==============================================\n');