EXPERIMENT 02

CHANNEL CODING AND ERROR CORRECTION.

clc;

clear;

close all;

%% STEP 1 : FIXED HAMMING CODEWORD

codeword = [1 0 1 0 1 0 1];

disp('Original Hamming Codeword:');

disp(codeword);

%% STEP 2 : EXTRACT ORIGINAL DATA BITS

% Hamming arrangement:

% [P1 P2 D1 P3 D2 D3 D4]

data = [codeword(3) codeword(5) codeword(6) codeword(7)];

disp('Original Data Bits:');

disp(data);

%% STEP 3 : INTRODUCE SINGLE-BIT ERROR

% Introduce error at data bit position 5

error_position = 5;

received = codeword;

% Flip bit at error position

received(error_position) = ...

1 - received(error_position);

fprintf('\nError Introduced at Position = %d\n', error_position);

disp('Received Codeword with Error:');

disp(received);

%% STEP 4 : SYNDROME CALCULATION

s1 = mod(received(1)+received(3)+received(5)+received(7),2);

s2 = mod(received(2)+received(3)+received(6)+received(7),2);

s3 = mod(received(4)+received(5)+received(6)+received(7),2);

fprintf('\nSyndrome = %d%d%d\n',s3,s2,s1);

% Calculate error position

error_detected = s1 + 2*s2 + 4*s3;

fprintf('Detected Error Position = %d\n', error_detected);

%% STEP 5 : ERROR CORRECTION

corrected = received;

if error_detected ~= 0

corrected(error_detected) = 1 - corrected(error_detected);

disp('Error Corrected Successfully');

else

disp('No Error Detected');

end

disp('Corrected Codeword:');

disp(corrected);

%% STEP 6 : DECODE DATA

decodedData = [corrected(3) corrected(5) corrected(6) corrected(7)];

disp('Decoded Data Bits:');

disp(decodedData);

%% STEP 7 : BER BEFORE CORRECTION

receivedData = [received(3) received(5) received(6) received(7)];

errorsBefore = sum(data ~= receivedData);

BER_before = errorsBefore/length(data);

%% STEP 8 : BER AFTER CORRECTION

errorsAfter = sum(data ~= decodedData);

BER_after = errorsAfter/length(data);

%% DISPLAY BER

fprintf('BER Before Error Correction = %.2f\n',...

BER_before);

fprintf('BER After Error Correction  = %.2f\n',...

BER_after);

%% =========================================================

% FIGURE 1 : SIGNAL REPRESENTATION

% ==========================================================

bitAxis = 0:7;

originalPlot = [codeword codeword(end)];

errorPlot = [received received(end)];

correctedPlot = [corrected corrected(end)];

figure('Name','Hamming Error Correction', 'NumberTitle','off');

%% ORIGINAL SIGNAL

subplot(3,1,1)

stairs(bitAxis,originalPlot, 'LineWidth',1.5);

grid on

xlim([0 7])

ylim([-0.2 1.2])

xticks(0:7)

yticks([0 1])

xlabel('Bit Position')

ylabel('Amplitude')

title('Original Signal : 1 0 1 0 1 0 1')

%% RECEIVED SIGNAL WITH ERROR

subplot(3,1,2)

stairs(bitAxis,errorPlot, 'LineWidth',1.5)

grid on

xlim([0 7])

ylim([-0.2 1.2])

xticks(0:7)

yticks([0 1])

xlabel('Bit Position')

ylabel('Amplitude')

title(['Received Signal with Error at Position ', num2str(error_position)])

%% CORRECTED SIGNAL

subplot(3,1,3)

stairs(bitAxis,correctedPlot, 'LineWidth',1.5)

grid on

xlim([0 7])

ylim([-0.2 1.2])

xticks(0:7)

yticks([0 1])

xlabel('Bit Position')

ylabel('Amplitude')

title('Corrected Signal : 1 0 1 0 1 0 1')

sgtitle('Channel Coding and Error Correction');

%% =========================================================

% FIGURE 2 : BER SMOOTH CURVE

% ==========================================================

figure('Name','BER Analysis', 'NumberTitle','off');

% Smooth x-axis

xCurve = linspace(0,1,100);

% Smooth BER curve from before to after correction

BERcurve = BER_before*(1-xCurve).^2;

% Plot smooth BER curve

plot(xCurve,BERcurve, 'LineWidth',1.2);

hold on

% Mark BER before correction

plot(0,BER_before,'o', 'MarkerSize',7, 'LineWidth',1.2);

% Mark BER after correction

plot(1,BER_after,'o',  'MarkerSize',7,  'LineWidth',1.2);

grid on

xlabel('Error Correction Process')

ylabel('Bit Error Rate (BER)')

title('BER Before and After Hamming Error Correction')

xlim([0 1])

ylim([0 0.30])

xticks([0 0.25 0.5 0.75 1])

xticklabels({'Before Correction', '','','','After Correction'});

% Display BER values

text(0.02,BER_before+0.015, sprintf('BER = %.2f',BER_before), 'FontWeight','bold');

text(0.98,BER_after+0.015,...

sprintf('BER = %.2f',BER_after), 'HorizontalAlignment','right',  'FontWeight','bold');

hold off

%% =========================================================

% FINAL RESULT

% ==========================================================

fprintf('Original Codeword  : ');

fprintf('%d ',codeword);

fprintf('\n');

fprintf('Received Codeword  : ');

fprintf('%d ',received);

fprintf('\n');

fprintf('Corrected Codeword : ');

fprintf('%d ',corrected);

fprintf('\n');

fprintf('Error Position     : %d\n',...

error_detected);

fprintf('BER Before Correction = %.2f\n',...

BER_before);

fprintf('BER After Correction  = %.2f\n', BER_after);